| - Annex 1 to the Contract according to DE-UZ 113 - Environmental Label for - „Low-Emission Floor Covering Adhesives and other Installation Materials“ |  | - Please use this - form ! |
| --- | --- | --- |

- Manufacturer (Applicant):

- Distributor (Label User):

- Brand/Trade Name:

- Applicant’s Declaration

- It is hereby declared that:

- the installation material meets the usual quality standards for serviceability of the corresponding product group,

- the installation material does not contain any substances and preparations as constituent components as listed under para. 3.1. of the Basic Award Criteria DE-UZ 113,

- the installation material is not classified as H400,

- the substances classified and labelled as hazardous to the environment -with either H410, H411 and/or H412- shall be limited in the installation material employing the calculation model M * 100 * H410 + 10 * H411 + H412 ≤ 11.0 %

- the type of installation material according to para. 2 is named on the container in conjunction with the product designation,

- no products containing plasticising substances from the group of phthalates or the group of organophosphates are added to the installation material (exceptions according to para. 3.4.2 apply),

- neither perfluorinated nor polyfluorinated chemicals (PFCs) are used (para. 3.4.3),

- the installation materials as well as the intermediates used do not contain as constituents any oxidizable fatty acids or oxidizable fatty acid esters,

- Organotin compounds are not used (The use of organotin compounds as catalyst for the curing reaction of SMP-based adhesives in accordance with the Recommendation of the Bfr XV Silicones shall be exempted from this requirement),

- advertising statements do not include any notes that would belittle the risks in terms of Section 25, para. 4 of the CLP Regulation 2008/1272/EC, such as, for example, “non-toxic”, “non-detrimental to health“ and the like,

- no advertising messages including name components or designations, such as „organic“, „eco“, „natural“, „fung“, „insect“ or „nano“ and the like are used,

- Exception: The comment „solvent-free according to TRGS 610“ shall be admissible.

- the components of the installation materials pursuant to para. 2 are listed in the Technical Data Sheets in conformity with VdL Directive on „Bautenanstrichstoffe“ (Building Coating Materials) - Vdl-RL 01/June 2004,

- the container text includes a prominent reference to the Technical Data Sheet and as to where the latter may be obtained as well as the manufacturer’s phone number at which consumers may obtain additional information,

- The container/package text and the technical data sheet shall include the following instructions in addition to the P-phrases required under the CLP Regulation (EC) No 1272/2008 in an easy-to-read form (other similar wording / P phrases may be used):

- „ Keep out of the reach of children“

- „Ensure good ventilation during and after use and drying“

- „Do not eat, drink or smoke when using this product“

- „In case of contact with skin or eyes, rinse immediately with plenty of water“

- „Do not allow product to reach sewage system, water course or soil“

- Information on cleaning the tools are provided on the container or the technical data sheet.

- „Give only empty containers to recycling. Dried product residues may be disposed of as domestic waste“

- „Product contains:………. (indication of the name(s) of the preservative(s) pursuant to Appendix C, para. 1); For information for allergic people, please call: ....………..“

- container text and Technical Data Sheet of SMP adhesives additionally include the following easily readable instructions/notes (similar wordings are admissible):

- „Methanol separates during hardening“

- „Ensure constant ventilation when using this product"

- „Ensure proper ventilation for several days after laying the flooring"

- „Wear protective gloves“

- container text and Technical Data Sheet of surfacers additionally include the following easily readable instructions/notes (similar wordings are admissible):

- Wear protective gloves“

- „Storage conditions: Keep in dry and cool place. Reseal container tightly immediately after use.“

- the Technical Data Sheet of cement-containing surfacers indicates the shelf-life and the container text indicates the minimum shelf-life.

- Additional instructions for GHS05 or GHS07-labelled products (other similar wording or P-phrases may be used) are displayed:

- „Always wear safety glasses.“

- „If adhesive or installation material gets into contact with your eyes rinse immediately with plenty of water and see an ophthalmologist.“

- „Always wear heavy-duty waterproof gloves to protect your hands.“

- „Always wear long trousers.“

- „Avoid prolonged skin contact with the adhesive or installation material. Rinse affected areas immediately with plenty of water.“

- „The longer fresh adhesive or installation material remains on your skin the greater the risk of severe skin damage.“

- „Keep children away from fresh adhesive or installation material.“

- The shelf-life shall be specified in the Technical Data Sheet of GHS05 or GHS07-labelled products and the best-before date shall be given on the container.

We declare that the products contain no preservatives

We declare that the products contain the following preservatives *

| Substance or mixture (common abbreviations can be used) | CAS-No. | concentration (weight-%) in products |
| --- | --- | --- |

* permissible are  in-can preservatives for aqueous installation materials listed in Appendix C " List of approved in-can preservatives" in the concentrations specified therein

Annexes

Annex 2:	List of used intermediates (excel sheet, please transmit electronically)

Annex 3:	Declaration of the Manufacturer/Distributor of intermediates (paras. 3.4.1 - 3.4.5)

Annex 4:	Safety datasheets of installation material and intermediates.

Test reports as confirmation of paras. 3.2 and 3.3

| Place: |  |  |  |
| --- | --- | --- | --- |
| Date: |  |  |  |

Legally binding signature /company stamp